INDUSTRY 4.O (DIGITAL ASSEMBLY PROJECT)

# DIGITAL QUALITY CONTROL AND ASSURANCE BY USING ARTIFICIAL INTELLIGENCE(AI)

STUDENT                                                     PROFESSOR: ALESSANDRO CHIARAVIGLIO

M.KASHIF RIAZ                                           COMPANY TUTOR: GIANLUCA ANGUZZA

THESIS WORK PLACE:      NV GROUP

ABSTRACT

The market lacks the Integration of Technologies and Lean thinking to real Assembly processes.

Although the technologies have been individually developed, there is a lack of systematic and logical

integration between these technologies and Lean Manufacturing Principles. The objective of the

thesis project is to develop a seamless integration using Industry 4.0 technologies and Lean

Manufacturing’s Process logics.

Experimenting with usual identification technology and applying it on assembly tracking and quality

Control in an innovative way. Using the potential of this technology with the mission of liberating

the operator from non-value added activities(MUDA), making the task less mentally stressful (MURI)

and controlling process irregularities (MURA).

ACKNOWLEDGEMENT

----------------------A BIG THANKS TO EVERY ONE------------------------

CONTENTS

PART A

INTRODUCTION INDUSTRY 4.O

MAIN GOALS

PRODUCT DESIGN

BILL OF MATERIAL (BOM)

MUDA, MURI AND GOLDEN ZONE

THESIS CLASSIFICATION-INITIAL SITUATION

EXPERIMENTATION OF INITIAL LAYOUT SITUATION

RESULTS-MUDA ANALYSIS

IMPROVED-INTERMEDIATE SITUATION

VISUAL INSTRUCTIONS

RESULTS-MUDA ANALYSIS

INTELLIGENT FACTORY

DIGITAL ASSEMBLY SKETCH

DIGITAL ASSEMBLY LAYOUT

MATERIAL FEEDING

DESIGN OF NEW WORKSTATION

REAL TIME WORK INSTRUCTIONS

IMAGE PROCESSING TECHNOLOGY-AI

SELF CONTROLLING TOOL

DIGITAL ANDON

SMART FACTORY RESULTS

PART B

LOGISTICS

INITIAL SITUATION-MATERIAL FEEDING

IMPROVED-INTERMEDIATE SITUATION

RESULTS

DIGITAL LOGISTICS 4.0- NEW DESIGN

DESIGN OF FULLY AUTOMATIZED OPERATIONS

DESIGN FOR AUTOMATIZED MATERIAL DELIVERY

ROBOTIZED SEQUENCING

AGV FOR AUTOMATIZED MATERIAL HANDLING

CONCLUSION

# INTRODUCTION INDUSTRY 4.O

Industry 4.O is the name given to the today trend of automation and data exchange in manufacturing technologies which includes cyber physical systems, internet of things(IOT), cloud computing and cognitive computing. Industry 4.O is commonly referred to as fourth industrial revolution. The term “Industry 4.O” was introduced by Germany very first time in 2011.

We can see many differences between a typical traditional factory and an industry 4.O factory (smart factory). In traditional factory environment the least product price is key to success and factories are running in this race to increase their profit margin. In this way many various data sources are present in the market to provide information about different aspects of factory while in industry 4.O factories have condition monitoring, fault diagnosis, systems and components are able to gain self- awareness and self predictiveness which provide management with more insight on current status of factory.

Industry 4.O is affecting and will affect in many aspects e.g productivity, reliability, machine safety, manufacturing, product life cycle, operator's education and skills etc.

MAIN GOALS

- Experimenting the concept of flow and its impact on productivity in assembly through the application of Industry 4.0

- Operations  knowledge and practicing the method and tools to create a dynamic process that is self-adapting to events

- Experimenting the route map for lean digital transformation

PRODUCT DESIGN

This thesis revolves around the making of one product design and assembly by traditional method and then by improvised and fully digitalized method (Industry 4.O).

We have one primary cart which consists of many variants. The components are two fixed brackets and four rotating brackets, one connecting bracket and one spring, nuts, bolts and jointers as per need. Then we have secondary frame which consists of horizontal and vertical bars and secondary frame is mounted over the primary frame. In the below picture you can see all the components. On left side we have components and on right side we have final assembly of cart.

We achieve final assembly after doing number of operations done by operators on assembly line. In our case we have 6 workstations. Each operator on workstation has instructions to follow. The operator on second workstation cannot work until the framework is received from first workstation and is same for all next work stations. The required material e.g wrench, key, nuts, bolts, screw driver, brackets, wheels etc are feeded by logistic operator. Safety instructions and all other required instructions are already have been given to operators to carry on smooth production.

BILL OF MATERIAL (BOM)

Bill of material is the list which contains all the components which will be used in production. Each component has its own name and number of components are mentioned.

MUDA, MURI & GOLDEN ZONE

MUDA is waste which can be of 7 types e.g transport, inventory, movement, wait, overproduction, over-process, and defects which affect productivity and efficiency. MURI is overburden and is related to ergonomics problems. We record the whole production process by a camera and then we analyze and find out the MUDA and MURI in process. We differentiate between non value added activities and added value activities.

Value added activities are those which contribute in the final cost and nonvalue added activities are those which do not contribute in the final cost.

GOLDEN ZONE

Golden zone is related to availability of materials in different zones. We have three types of materials namely A class material, B class material, C class material. A class material is that which is expensive and heavy, B class material is less expensive and heavy as compared to A class material while C class material is non expensive and very light in weight. In our product A class material consists of moving and fixed brackets, B class material consists of tires of different colors while C class material consists of nuts and bolts. A class material and B class material both are feeded by logistic operator while C class material is already present at workstation.

AA zone: The material which is easily accessible in the range of operator's hands.

A zone: To pick material an operator has to extend his hands.

B zone: To pick material an operator has to take 2-3 steps.

C zone: To pick material an operator has to turn backward.

D zone: To pick material an operator has to turn backward and then to take 2-3 steps.

THESIS CLASSIFICATION

This thesis is classified into 3 situations.

1-Initial situation

2-Imroved-intermediate situation

3-Industry 4.O situation

1-INITIAL SITUATION

In initial situation frame work is on workbench. The place is assigned for an operator. The box which contains tools and materials is present on the floor. The paper instructions for operator are present in theoretical form and are not balanced. One operator has to do more operations while the other operator has to do less operations. Theoretical instructions are not so clear and well explained because two operators can do 1 operation in two different methods.

As said earlier the box which contains tools and materials is present on floor so operator has to move, bend and then to transport tools are materials on work bench. The movement, transporting and then to search material from the box are counted as MUDA because such type of activities consume some time so are counted as wastes. When operator bends to pick material from box can have ergonomics problems which are referred to MURI.

EXPERIMENTATION OF THE INITIAL LAYOUT SITUATION

With the help of camera we record the whole process of initial situation and later we analyze and find out wastes (non-value added activities) and ergonomics problems faced by operators. In the picture it is visible that an operator has to walk and then to bend and then to material transport is happening which are all wastes.

Non value added activities (NVA) do not add any value in the final product which is direct loss for the company so the walking, material handling, bending, material transport all are non-value added activities which increase the cycle time due to which productivity reduces. So it means that company is selling less than target because of wastes and losses.

RESULTS-MUDA ANALYSIS

We do MUDA analysis with the help of recorded video and find out value added activities(VA) and nonvalue added activities (NVA). Non value added activities can be waiting, handling, over production, repair, inspection, nonvalue added but necessary, material transport, walking, waiting in process.

We can see that in a process we have 49% value added activities (mentioned in green color) and 51% nonvalue added activities (51%).

We make Yamazumi chart of every workstation and for each workstation we find out value added activities(VA) and nonvalue added activities(NVA).

Main losses which we found are material transport and walking because these activities consume time due to which cycle time increases and productivity decreases.

2-IMPROVED-INTERMEDIATE SITUATION

This is improved situation as compared to initial situation. We have less losses and wastes in it. MUDA and MURI is less if we compare with previous situation. We eliminate non value added activities(NVA) which were because of material transportation and movements between the workbench and materials and tools.

In this improved situation material is being feeded in golden zone at frontal position to avoid material transport and walking.

Tools are present on the workbench instead of floor. Instead of walking to pick material and then to transport material we use manual flip flop to take material and to leave empty boxes. We use pneumatic pedal by which flip flop gives us material and empty boxes go away. In the following pictures we can have an idea.

VISUAL INSTRUCTIONS

In this improved situation we have visual instructions in the form of pictures which are more easy to understand and easy to follow. Any chance of mistake is eliminated. Operators can easily follow the visual instruction to perform their work.

Visual Work Instructions (standardized and rebalanced) are displayed on each workstation. Operators have to follow those instructions to complete the operations

RESULTS-MUDA ANALYSIS

In this situation we have better results than previous because non value added activities are reduced while added activities have been increased. Now we have 64% value added activities while before we had 49%  non-value added activities are 36% while before were 51%.

CYCLE TIME AND PRODUCTIVITY

In improved situation our cycle time is reduced by 25% and productivity is increased by 33% as compared to initial situation. It is because losses and wastes have been reduced.

3-INTELLIGENT FACTORY (INDUSTRY 4.O)

It is fully automatized, digital tracking, zero handling, material in golden zone, synchronized material feeding. Losses and wastes are highly eliminated and cycle time decreases due to which productivity increases. The operator conducts Value added activities only, all the NVAA activities and activities with MURI and MURA are automated.

DIGITAL ASSEMBLY SKETCH

In our production line we have one supermarket. AGV is being used for the transportation of material from supermarket to production line. Workbenches are conveyers which are fully digitalized and has RFID and digital Andon.

DIGITAL ASSEMBLY-LAYOUT

Layout of the conveyers in line: the work benches are configured as in fig

MATERIAL FEEDING

Material feeding is fully synchronized by following the JIT (Just in time), JIS (Just in sequence), Digital Kanban, Two bin. Following picture shows that which material is being feeded by which concept.

DESIGN OF NEW WORKSTATION

We design new workstation which is best as compared to previous cases because MUDA, MURI, wastes and losses are eliminated. Instead of manual flip-flop we use electric flip-flop. Tools are positioned according to real need at each location. Electric screw driver instead of on floor or workbenches is hanging which is easy to use.

REAL TIME WORK INSTRUCTIONS

- Work instructions are displayed on the monitor automatically synchronized with the production sequence as scheduled by the MES system.

- Work instructions are in the form of videos installed in monitors for each workstation.

- Real time work instructions are standardized, reduce defects, increase efficiency, traceability, improve quality, decrease training time, process compliance, quick and easy work instructions to follow.

- An operator at workstation has to follow the work instructions from screen.

- Each workstation has its own work instructions which are saved in screen.

IMAGE PROCESSING TECHNOLOGY-ARTIFICIAL INTELLIGENCE

- Image processing/recognition technology is being used in quality control by following the programmed instructions for process. In case of right step and wrong step we get notification. At the end of process we are being informed

- The system notifies if a wrong operation is performed and stops the line (Error proof)

By the help of this new technology we achieve maximum efficiency, higher productivity, process compliance, improved quality, easy to follow. When there is any error the system stops and operator cannot proceed further until problem is solved so at the end we get error proof product which is good otherwise in traditional production we get defects after manufacturing due to which we have to root cause analysis to find out the cause of defect which consumes a lot of time and defect causes money loss and time loss.

SELF CONTROLLING TOOL

Quality is being controlled by control screw driver

- Torque and angle is measured by controller during the process of bolts    fastening

- The number of fastening is checked

- Wrong fastening is notified and the system stops the process (ERROR PROOF)

Torque can be from 0,01 Nm to 49 Nm. This screwdriver is designed for demanded tightening process, complete line of hand held and this toll provides highest productivity and manufacturing quality at optimized cost.

This tool increases efficiency, productivity and quality while reduces time duration of tightening, errors, any defect during tightening. This tool is also used position control and sequence control.

DIGITAL ANDON

Andon is a manufacturing word which is referred to a system which notifies the management of process if there is any error or any problem during the process. The alert can be activated by pushing the button which notifies in terms of audio, colours or alarms. Andon is a key element of Toyota Production System (TPS) and part of lean manufacturing. It provides the worker an opportunity to stop the process if any defect is found so that supervisor could come to resolve the issue.

On LCD screen all details of production are visible. For example the target, no of pieces made and still in progress pieces are all visible. Total number of workstations are 6 and each workstation has 3 colours  which are red, green and yellow.

The background of workstation on LCD  becomes red if the process is stopped and need is required at workstation. Supervisor or responsible person comes and then they do root cause analysis to remove that problem from the process. The background of workstation on LCD becomes yellow if any help is required but process is not stopped and still working. If there is not any problem the background becomes green.

Andon lights are attached to each workstation to know the status of workstation. As discussed already it has three colours green, yellow and red which are turned on according to the real situation of workstation.

SMART FACTORY-RESULTS

Because of latest technology used we have better results. Non value added(NVA) activities are very much reduced to just 19% while value added activities are increased to 81%.

Cycle time is reduced by 55% and productivity is increased by 60%.

PART  B

LOGISTICS

Again in logistics we have three situations which are related to operations.

1-Initial situation-material feeding (Related to initial situation of operations)

2-Imroved-intermediate situation

3-Digital logistics 4.O

So now we will discuss one by one.

- Initial situation-material feeding

In this case material is feeded by logistic operator by doing patrolling. Boxes for each operator are on floor. Logistic operator feeds each box by reaching at each workstation. From pictures we can see that logistic operator has to a lot of movements and extra handling which are considered as MUDA and due to bending logistic operator experiences MURI. In this situation we have non synchronized material feeding.

2- Improved-intermediate situation (Related to intermediate situation of operations)

This is improved and improvised situation as compared to initial situation. We have Heijunka box which has specified places for different materials due to which operator does not waste his time in search of material. Logistic operator does manual feeding at front position of each workstation due to which operator picks the required material without any extra walking and movement due to which non value added activities are eliminated.

Logistics operator prepares the Heijunka boxes by picking materials from supermarket and then feeds that material at frontal position and picks empty boxes.

Instead of bending again and again to feed the boxes on floor, the logistic operator just in one operation feeds material due to which non value added activities are highly reduced.

RESULTS

After improvement as compared to initial situation we eliminate MUDA (extra handling waste, material transport waste) and MURI (ergonomics problems) due to which cycle time of process is reduced and it leads to higher productivity.

In our case cycle time is reduced by 25% which leads to increase in production by 33%.

DIGITAL LOGISTICS 4.O – NEW DESIGN

- DIGITAL PULL: The line is fed in function of and synchornized to the  real consumption of the line by digital sequence control. which follows the Takt time.

- SYNCHRONISED DIGITAL SEQUENCING: Right Material is available with the help of MES in right quantity, right sequence when and where needed.

- SELF ADAPTING: The right material right sequence is adapted in real time to the instantaneous state of the line and quality problems.

- This results in Reduced inter-operational Stocks.

DESIGN OF FULLY AUTOMATIZED OPERATIONS

- Synchronized material front feeding

- Automatic and synchronized information flow with production scheduler

- Automatic Digital Just In Sequence (JIS) through robot kitting

- Synchronized subassembly station

- AGV does logistics

DESIGN FOR AUTOMATIZED MATERIAL DELIVERY

In digital logistics 4.O instead of manual we have electric flow racks which are fully automatized. Digitally material is feeded at frontal position and empty boxes are collected (presence of sensors). Moving from manual flow racks to digital flow racks we have decrease in cycle time and increase in productivity.

ROBOTIZED SEQUENCING

In digital logistics 4.O instead logistic operator, kitting is done by robot. Kitting by robot provides a highly increase in productivity and efficiency of assembly operations. For this first robotic programming has to be done according to needs of operation.

AGV FOR AUTOMATIZED MATERIAL HANDLING

Before logistics were done by operator on racks but in logistics 4.O we use automated guided vehicle(AGV) which eliminates the human involvement because of fully digitalized and it works once its programming is being done. AGV optimizes workflow, increases productivity and reduces costs. AGV is web based easily operated by computer or mobile to eliminate bottlenecks and downtime throughout the facility.

CONCLUSION

If we see all pros and cons the initial situation costs very less but has more losses, wastes and ergonomics problems while intermediate situation which is improved as compared to initial situation costs more than initial but wastes, losses and ergonomics problems are reduced. The best situation is industry 4.0 which costs a lot as compared to previous situations but it has less cycle time and high productivity which is beneficial.

| SITUATION | MURI & MUDA | CYCLE TIME | PRODUCTIVITY |
| --- | --- | --- | --- |
| INITIAL | VERY HIGH | LONG | LOW |
| INTERMEDIATE | REDUCED | REDUCED | A BIT INCREASED |
| INDUSTRY 4.O | VERY MUCH REDUCED | VERY MUCH REDUCED | A LOT INCREASED |