# Introduction

The explosive science seems to be very clear nowadays, since we are using them every day in thousands of applications. However, the science of explosive is still very young, even though it is well known since over thousand years, and we are only beginning to understand the nonlinear interaction between chemistry and fluid mechanics that produce the rapid energy release known as detonation.

This science field is basically a coupling of chemistry and fluid mechanics. Nevertheless, the fluid-mechanical phenomenon of detonation is reasonably well understood, though, the detailed chemical reactions and thermos-mechanics that cause a detonation are still largely a mystery. This is due to two main reasons, first of all we have to be aware that measurements in the interior of a detonating explosive are extremely difficult for a lot of reasons, such as the lack of proper and available (economically speaking) gauges, the difficulty to get data, the tough environment developed by the detonation (high energy, high temperatures, high pressures, high wave velocity, short time); the second reason is the most disappointing, so true though.

Most explosive applications are in the fields of excavation, mining, or conventional munitions. Thus, it is possible to say that all these fields have well-established, albeit crude, “rules of thumb” as regards quantities and configurations of explosive required to accomplish the task, as we saw during our University courses.

On the bright side though, in the last decades the need of understanding the details of energy release in explosives has been increased rapidly. Explosive systems are, nowadays, demanded that function with an extreme precision and efficiency and, last but not least, maximizing and improving the safety. These needs have allowed to open a new chapter to the research in this ancient field, where, through new technologies would be possible to get deep in these problems. For instance, now is possible to use very powerful tools, such as Numerical simulations, Finite Analysis Methods, Computational Fluid Dynamics software and so on and so far.

The purpose of this study is to analyse the data that we have got so far from different experiments in different environments, which they will be shown afterwards, and see how we can link this data to the problems mentioned here above. What we are saying is that throughout the measurements of the pressure shock waves, which we have collected by means of pressure gauge sensors of different types, we would like to give answers to different questions, and in some case, just to stimulate the mind of the reader in order to let him/her understand the huge potential of this field research.

Now, to an explosive system is required, as we said previously, extreme precision, efficiency and safety.

These three themes are very close to each other, especially in blasting engineering, namely, for mining and excavation purposes. Let us do some examples of the thousands possible applications.

In mining, as well as in excavation, it is well known the “decking technique”, which refers to the axial or longitudinal decoupling of an explosive column by dividing it into decks spaced by gaps. These gaps are usually left with air although other inert materials such as sand or drilling detritus have been used. For a given charge length, decking improves energy distribution along the length of the hole; however, it will tend to generate an excess of fine from those sections of borehole in contact with the explosive and a poor fragmentation from those sections which are not. A problem arises from the need to prime each individual explosive deck introduces an element of risk as a consequence of explosive and/or initiator malfunction due to shock that could lead to misfires or propagation failures.  A scheme is shown in Figure 1, besides, it is common knowledge that in a particular case of Air Decking is possible to avoid subdrilling, increasing the precision and the control of the blast, reduction of vibration and flyrocks.

Figure 1-Decking technique, different typologies

One problem could be: how can I get close to the explosive in decking technique, avoiding the sympathetic detonation and/or malfunctioning of the other detonators?

Remind: in this technique would be possible to delay each charge or to blast them simultaneously, it depends on the required effect.

The pressure thresholds for malfunctioning and sympathetic detonation are known and published according to the kind of explosive as well as the type of detonator and the brand. What we do not know is how the propagation of the shock wave behaves through the medium.

Another problem could be achieved in the other dimension, considering the later side. For instance, in underground environment, while we are dealing with one free surface. Let’s take an excavation by means of parallel holes, Figure 2.

Figure 2- Parallel holes pattern

Having a look at the zoomed square on the right, the burn cut, it is known that in this kind of blasting, its efficiency depends on the delay sequence of blasthole detonation, indeed delayed detonation improves loadability of the entire cut, contributes to a better strata control and reduction of blast-induced vibrations. So still, how close could we drill the holes (the least spacing) in order not to influence the other detonators in the around holes?

Before answering these questions we must pass through ??????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????

# 1 Explosives

## Introduction

What is an explosion?

It is possible to define an “explosion” as a phenomenon in which there is a significant release of energy in a very short period. This term could be assigned to different kind of phenomena, for instance we could mention the explosion into a engine cylinder, explosion of a pressured tank and obviously, explosion of a bomb charged with detonating or deflagrating explosive.

However, in this report we are going to focus in the last case of our examples up here, namely, detonating explosives. Thus, to make it “usable”, as well as “suitable”, an explosive it is necessary that the explosion reaction takes place only when the explosive material is subjected to an impulse (heating, impact and so on). This impulse is called “ignition”. Furthermore, it is necessary that the reaction, quoted up here, once ignited, doesn’t interrupt and thus it must be kept until the total decomposition of the charge. At this point it is possible to attach two of the many definitions of explosive:

“Substance (compound or mixture) in a condensed state (solid or liquid) that, after a given localized impulse (ignition), it reacts in an exothermic manner, in a very short time, with all the final products, or nearly all of them, at the gaseous state, at the reached reaction temperature” (R. Mancini, M. Cardu, Scavi in roccia, gli esplosivi, 2001)

According to Persson: “From a practical point of view, explosives are simply materials that are intended to produce an explosion, i.e., lo have the ability to rapidly decompose chemically, thereby producing hot gas which can do mechanical work on the surrounding material" (P.A. Persson , R. Holmberg & J. Lee, Rock Blasting and Explosives Engineering, 1994).

Nowadays it is well known that, the explosive world is divided in two big families: deflagrating explosives and detonating explosives. In this essay the attention will be about detonating explosives, since in mining engineering, as well as excavation engineering, these compounds play a fundamental role in blasting operations.

Therefore, the following paragraph will be about where the differences are for these two families and their features.

## Two faces of Explosion: deflagration and detonation

Upon ignition, a combustion wave propagates away from the ignition source. Combustion waves transform reactants into products, releasing the potential energy stored in the chemical bonds of the reactant molecules, which is then converted into internal (thermal) and kinetic energy of the combustion products. Large changes in the thermodynamic and gas dynamic states occur across the combustion wave as a result of the energy released. The gradient fields across the wave generate physical and chemical processes that result in the self-sustained propagation of the combustion wave.

Generally speaking, there are two types of self-propagating combustion waves: deflagrations and detonations. Deflagration waves propagate at relatively low subsonic velocities with respect to the reactants ahead of it. As subsonic waves, disturbances downstream can propagate upstream and influence the initial state of the reactants. Thus, the propagation speed of a deflagration wave depends not only on the properties and initial state of the explosive mixture, but also on the rear boundary condition behind the wave (e.g., from a closed-end or an open-end tube. See 6th point in Deflagration characteristics down here). A deflagration is an expansion wave where the pressure drops across the reaction front, and the combustion products are accelerated away from the wave in a direction opposite to its propagation. Depending on the rear boundary condition (e.g., for a closed-end tube the particle velocity is zero in the products), the expansion of the products causes a displacement of the reactants ahead of the reaction front. Thus, the reaction front propagates into reactants that are moving in the direction of propagation. The deflagration speed (with respect to the fixed laboratory coordinates) will then be the sum of the displacement flow velocity of the reactants and the velocity of the reaction front relative to the reactants (i.e., the burning velocity). Compression waves (or shocks) are also formed in front of the reaction front as a result of the displacement flow. Thus, a propagating deflagration wave usually consists of a precursor shock followed by the reaction front. The strength of the precursor shock depends on the displacement flow velocity, and hence on the rear boundary condition.

The mechanism by which the deflagration wave propagates into the reactants ahead of it is via diffusion of heat and mass, according to the Vielle’s law (eq. 1).

Let’s have a practical example taking a stick of a rocket propellant, say, made of nitrocellulose (NC) and nitro-glycerine (NG; i.e., a “double-base” propellant). When it is ignited at one of its ends, it burns rather vigorously, layer by layer. The salient points of a deflagration process are indicated in Figure 1:

Figure 3- Deflagration of a Propellant Stick.

Deflagration has the following characteristics:

- The propellant burns layer by layer.

- There are different zones existing above the burning surface as shown, varying in temperature, pressure, concentration, and composition of gaseous products.

- The hot gaseous products emerge away from the regressing surface.

- The most important characteristic of deflagration is that the rate of deflagration (or the rate of recession of a burning surface, often expressed in millimetres per second at a given pressure) is much below the sonic velocity of the material (i.e., the velocity of sound through the propellant material).

- The process of deflagration is sustained by thermal feedback from the flame to the surface temperature by means of conduction, convection, and radiation.

- The rate of regression (or burning rate, r) heavily depends on the pressure of the surrounding gases (P), and, according to Vielle’s law, a double-base propellant nearly obeys the equation:

r=bPn (1)

where n is the index of the burning rate, which describes the pressure dependence. For                  deflagration n <1, however, this value can increase to n >1 for detonation, we are talking about one particular case, DDT (Deflagration-to-Detonation, it is fair to quote this case since it has to be known that a deflagrating explosive under certain conditions, such as confined environment as said before, it can undergo in detonating reaction, nevertheless, it is not going to be treat in this essay), and b is the temperature dependent coefficient (b=f(T)).

Hence, a detonation can result from either a continuously accelerating combustion (as it was said before, DDT) or from a shock (using a primary explosive in a detonator to initiate a secondary explosive, unfortunately we have to take for granted the concept of primary and secondary explosive), in the first case it will be valid what was said previously and summarized in the 6th point here above, (Eq.1). Whereas the second we assume that an explosive is subjected to a shock-wave.

Generally speaking, a shock wave is a disturbance propagating at supersonic speed in a material, accompanied by an extremely rapid increase in pressure, density, and temperature. When a large amount of energy is suddenly released in a very limited space, it produces a shock wave. It may be mechanical energy, electrical, or chemical energy. The shock-wave caused by a detonation is called a “detonation wave.” Therefore, a detonation wave is a shock-wave, but all shock waves are not detonation waves. When a shock wave is not sustained, say, by continuous feeding of energy (as in case of a detonation wave, in which continuous evolution of thermochemical energy and gaseous products behind the shock front keeps feeding the shock-wave), it loses energy because of viscous dissipation by the surrounding medium and it degenerates into a sound wave.

The detonation process in an explosive requires a shock wave for initiation. This shock-wave that initiates a detonation, in our case, is originated by detonators, namely a detonating primary explosive that initiates a secondary explosive. This shock-wave compresses the secondary explosive enough to cause the temperature to increase to higher than the decomposition temperature through adiabatic heating and the explosive material that is directly behind the shock-wave front reacts. During this reaction the shock-wave accelerates as a result of the strongly exothermic reaction of the secondary explosive.

The shock-wave moves through the explosive under constant acceleration until it reaches a stationary state. The stationary state is reached, when the free energy released through this exothermic chemical reaction is the same as the energy which is released to the surroundings as heat plus the energy which is necessary to compress and move the crystal.

This means that when these chemical reactions occur with the release of heat at constant pressure and temperature, the propagation of the shock-wave will become a self-sustaining process.

Now, let us see at what happens when the process of detonation occurs in an explosive.

The following figure, Figure 2, shows a cylindrical stick of trinitrotoluene (TNT) which is detonating.  It is possible to note different characteristics during the process of detonation.

Figure 4- Detonation of an Explosive.

- The detonation is accompanied by the production of a shock wave.

- The wave front of the shock wave has a high temperature and pressure gradient (shock-zone), which instantaneously initiates chemical decomposition of the shocked explosive layer of the undetonated explosive.

- The chemical reaction of explosion is completed in the chemical reaction zone. The shock zone is very narrow (about 10-5 cm) as compared with the chemical reaction zone (varies from 0.1 to 1.0 cm), and both of these zones together form the detonation zone.

- The gaseous products flow in the same direction as that of the propagation of detonation.

- The rate of propagation of the detonation front (velocity of detonation (VOD)) is more than the sonic velocity of the material (i.e., the velocity of sound in undetonated TNT). The VOD varies from 1500 to more than 9000m/s for different explosives.

# Pressures

## Introduction

To understand the phenomena which develop inside a borehole in the simplest way, it is going to be useful to quote some statements taken by the Nobel Prize William Henry Bragg, as well as quoted by Prof. Mancini and Prof. Cardu in their book. It will give us an idea very clear making simple examples.

Let’s imagine that the little balls in Figure 4 below are the gas molecules

Figure 5- Bragg's Scheme.

As showed in Figure 4, the analogy is made by means of a billiard with a mobile edge, which can be moved upwards or downwards. If the molecules are going to move the edge because of their movement, we will have that the work is carried out by their impacts agaist the edge, therefore we will notice a loose of their own energy. However, if we are going to move the edge towards them (upwards), we will have that the work would be carried out by the edge, thus in this way the molecules are going to get energy.

Now, in the explosion case, it is easy to deduce that in the first place the balls are stopped, thus there is no movement and they are bonded to each other by chemical bonds, which indeed they form the explosive molecules. These molecules, as described in the previous paragraph, need “initiation energy” for starting the explosive reaction, namely the breakege of their own bonds.

The pressure (thurst against the edge) depends on:

- the numbers of balls present on the table (density)

- the speed of the balls (temperature)

In the following figure, Figure 12, it is possible to see a good sketch, very easly understandable; each molecol is linked to each other through a little wire which keeps a spring compressed, if we break this little wire using, for instance, a match (which is our initiation), the spring will release its own elastic energy and the ball will start moving according to this.

Figure 6- Sketch of the initiation

## Explosion pressures

It is important to remark the term “pressures”, since we are talking in plural in full knowledge of the facts. From a damage control point of view, the most important parameters related to the performance of an explosive are the detonation pressure, the explosion pressure and the borehole pressure. The following figure, Figure 13, makes the concept clearer.

Figure 7- Definitions of some specific pressures on the pressure history in a blasting hole

Let us regroup the pressures mentioned before under two families: pulsed pressures, due to a “dynamic load” and “quasi-static” pressures, also known as “chemical pressure”, due to the containment of the gases in a little volume. The difference between those two pressures is well explained in the following picture, Figure 14, where, in a simple way is possible to diversify each other.

Figure 8- Differences among the pulsed load and quasi-static load.

Considering a mass of 1 kg in Figure 13, which is hanged at a certain height above a dynamometer. When we cut the rope that is holding the hanging mass, we notice that mass will fall in a short time, Δt, till it stops. The dynamometer in that short time, will record an heavy load (exactly the pulse) and it will be followed by a stedy-low load, which simply is the weight of the mass.

Now, the dynamic load is the force necessary to annul the mass velocity (Δv) in a the shock time Δt, namely : Fd=M∆v∆t.

Instead, the steady load is just Mg.

Now, the analogy is well clear, namely, the pulse represents the dynamic pressure, instead the other, represents the chemical pressure, which can be conceived as Explosion and Borehole pressure, since those two appear right after the detonation phenomenon (depending on the design of the blast, as you can read in 3.1.3 below).

### Detonation Pressure

The detonation pressure referes to the pressure developed right behind the detonation front before any gas expansion takes place. This pressure is usually referred to as the Chapman-Jouget state, or CJ state for short. For ideal explosives, full chemical reaction (complete oxidation) has been achieved at this state; however, in the case of non-ideal explosive product, the chemical reaction continues beyond the CJ state and into the expansion zone of the reaction products. From the conservation of momentum equation describing shock phenomena, saw in paragraph 2.3.2, the detonation pressure is given by:

Pd=ρDu (2)

Where ρ is the original density of the explosive, D the velocity of the detonation font and u the particle velocity behind the detonation front. The equation (6) can be also expressed as:

Pd=ρD2(1+γCJ) (3)

Where γCJ is the coefficient of adiabatic expansion at the CJ state. For condensed high-density explosives (ρ between 1.0 g/cm3 and 1.80 g/cm3), γCJ is approximately equal to 3. By replacing this value in equation (7), the well-known expression used to estimate detonation pressure is obtained:

Pd=ρD42(4)

### Explosion Pressure

The explosion pressure is defined as the pressure exerted by the expanding detonation products when they occupy the original volume of the explosive charge. If we assume a γ-law equation of state with a value of γ remaining constant throughout the expansion process from the state CJ state to the original volume of the charge, then the explosion pressure equals one-half the detonation pressure (Zerrill, 1981). Therefore, using equations 7, 8, the corresponding pressure expressions are obtained:

Pe=ρD22(1+γCJ) (5)

Pd=ρD42 (6)

Other several formulae were also developed to estimate explosion pressure, however is not in our interest to show them.

### Borehole Pressure

The borehole pressure refers to the pressure exerted by expanding detonation gases against the wall of the borehole. When an explosive charge is bulk-loaded into the blasthole, the borehole pressure and the explosion pressure equate, however, when decoupled explosive charges are used, the borehole pressure will have a reduced value which depend on the extent to which the products of detonation are allowed to expand. Assuming the expansion process from the explosion state to the borehole state is adiabatic (i.e. PVγ =constan) with a constant γ coefficient, the borehole pressure (Nie, 1999) will be given by:

Pb=Pe(LeLbrerb)2γ (7)

In other words, the borehole pressure is function of the explosion pressure Pe, the ratio of charge to blasthole radius re/rb, the ratio of charge to blasthole length Le/Lb and the value assigned to γ.

### Summarizing Pressures Concepts

To sum-up  the previous concepts, since we retain they are very important, we will take into account the following definitions and dependencies.

First, the detonation pressure is a function of the explosive density, as well as the velocity of detonation and the coefficient of the adiabatic expansion (usually assigned a value of 3); its amplitude is determined by the conservation of momentum equation (2.3.2). This conservation equation is appropriate for ideal conditions involving full chemical reaction at the CJ state, behaviour that is best represented by condensed high-density explosives such as most military formulations. However, commercial explosives exhibit a higher dependence on external and internal factors such as charge diameter, heterogeneity of the mixture and degree of confinement. These factors will affect the explosives’ ability to reach full chemical reaction at the CJ state and, as a result, they will exhibit the non-ideal behaviour that characterizes most commercial products.

The explosion and borehole pressures are both functions of the detonation pressure; thus, all the comments mentioned for the detonation pressure also apply. The amplitude of the explosion pressure is usually taken as being half the value of the detonation pressure; otherwise, the empirical relationships presented can be used to estimate it. The borehole pressure, in turn, is expressed as a function of explosion pressure and estimated assuming a constant value for specific heat ratio during the gas expansion in the annular space created by decoupled charge.

# Shock waves

## Introduction

Before going into detail, we have to be aware of the way in which shock waves travel and behave in a medium. Therefore, we would like to point out some mechanical models and then proceed to develop the basic equations that describe dynamic uniaxial strain, which is the only one direction where the shock travels.

## Qualitative description of a shock wave

Before getting through the shock-wave topic, it would be better make a little refresh of compression characteristics of most materials. It is more than familiar the typical stress and strain curve, the most common case is shown in Figure 3, linear behaviour.

Figure 9- Linear Stress&Strain behaviour.

As we know, almost all materials behave linearly. That is, the strain (amount of distortion) produced in a material is directly proportional to the stress placed on the maerial. This linear behaviour holds until a point at which the material, if release, will not return to its exact original shape or dimensions. This point is called the yield point or elastic limit. When we strain a material beyond its elastic limit, we cause plastic deformation.

Now, in shock behaviour, we will only consider compressive stress and strain. Also, to keep the mathematics and models simple, let us only consider uniaxial compressive stress and strain. This means we will be studying these effects along only one axis of a material. We will assume the dimension of the material perpendicular to the strain axis to be infinite. This assumption means that the systems we will study have no edge effects.

Taking the same material but applying a much, much higher level of stress, it will behave kind of differently, we are going to notice an elastic region which will be followed by a combined Elastic-Plastic region, till finally then it will reach the plastic region, in this case would be very close to a fluid behaviour. In the study of strong shock-waves, this is the region we can most easily evaluate, the region in which we work most with explosives.

In the elastic region, the sound velocity in the material is constant. The sound velocity, C, is proportional to the ratio of the change in pressure with change in density:

C=dPρ

This means that in the elastic region, the pressure and density are linearly related. Beyond the elastic region, the wave velocity increases with pressure or density and the ratio P/ρ is not linearly proportional. Wave velocity continues to increase with stress or pressure throughout the region of interest. Thus, up to the elastic limit, the sound velocity in a material is constant. Beyond the elastic limit, the velocity increases with increasing pressure.

Now, considering the following figure, Figure 4, at point C the pressure is low; therefore, Cc, the wave velocity, is low. Also the particle velocity, uc, the speed to which the material locally has been accelerated is fairly low. Therefore, the velocity of the pressure wave is low, U=C0+u0. At point B, the wave velocity is higher than at point A, because as was just pointed out, the wave velocity increases with increasing pressure (we are above the elastic limit). The particle velocity is also higher, since the higher is the pressure, the higher is the particle velocity. Thus, the pressure wave at point B is traveling faster than point C. The same argument holds for point A, that has a faster wave velocity than point B.

Figure 10- Pressure wave at high pressure.

The effect is shown in Figure 5, where we see the wave continues to get steeper and steeper at the front until it approaches a straight vertical line.

When the wave assumes this vertical front, it is called a shock wave.

Figure 11- Shocking-up of a pressure wave.

Notice that there is not a smooth transition from matter in front of the wave to matter behind the wave. The material “jumps” from nonshocked to shocjed state. This is called a discontinuity.

## Attenuation behind shock waves

A smooth front pressure disturbance will “shock-up” because the wave speed increases with increasing pressure. Having a look at a square wave in Figure 12, suppose that it is traveling through a material.

Figure 12- Square-wave shock.

The front of this wave is already a shock; so let us examine the back of the wave. The material front of point A is at high pressure. It is also traveling at particle velocity u, and is at high density ρ. The velocity of the back of the wave is the sum of particle velocity and wave speed, as described before. Now, since the back is moving into a higher-density region than the front and is also encountering a faster material or particle speed than the front, it is traveling faster than the front. It therefore tends to catch up rapidly with the front. So as the shock wave travels along, the back side, or rarefaction wave, smears out the back and eventually catches up to the shock front. Still traveling faster than the front, the upper parts of the rarefaction wave now start “whittling down” the pressure in the front, until eventually the pressure is reduced to the region of elastic behaviour, and the shock has decayed to a sound wave; the whole process is shown in Figure 13 below.

Figure 13-  Attenuation of a shock-wave shock.

To sum it up, it has been said that:

- Shock waves occur when material is stressed far beyond its elastic limit by a pressure disturbance.

- Because pressure-wave velocity increases with pressure above the elastic limit, a smooth pressure disturbance “shocks-up”.

- Because the rarefaction wave moving into the shocked region travels faster than the shock front, the shock is attenuated from behind.

What we are going to analyse in this paper, it is right this shock-wave behaviour, where the rarefaction is shown in Figure 14 below, which shown one of our result according to the Katsiabanis’s calibration curve that will be explained later.

Figure 14- Pressure curve.

## Shock wave equations

Remember the description of a shock, where the front was referred to as discontinuos, then the original states of particle velocity u0, density ρ0, internal energy E0, and pressure P0, suddenly change across the shock front. They do not change gradually but they discontinuously jump from unshocked to shocked values. Now the problem, we have five variables to deal with; so we will need five relationships or related equations to solve for all those variables.

The first three relationships can be derived from the fact that we must conserve mass, momentum, and energy across the shock front. These balances fo not depend upon a process path but simply upon the initial and final states of the material in question. These three equations take the names of  “Rankine-Hugoniot jump equations”.

### Mass balance

As we know, the mass balance implies that we are neither creating nor destroying mass. What goes in comes out.

m=ρv

Where m is the mass, ρ is the density and v is the volume.

v=AL

A is the cross-sectional area of the control volume

L is the length, which in this system can be found as the distance a particle travels relative to the shock front, thus is the velocity relative to the front times the time it took to travel that length, which means:

L=t*velocity,relative

According to the statements written above, we will have mass entering (subscript 0) and mass leaving (subscript 1):

ρ0AL0U-u0=ρ1AL1(U-u1)

Rearranging the equation and considering v=1ρ we can write:

ρ1ρ0=U-u0U-u1=v0v1 . (8)

### Momentum Balance

The momentum balance implies that the rate of change of momentum, for the control mass to go from the state before the shock to the state after the shock, must be equal to the force applied to it.

The force applied is simply the pressure difference across the shock front times the area over which it is applied, our control volume cross-sectional area

F=P1-P0A

The rate of change momentum is:

r=(mu1-mu2)/t .(9)

Combining the equation (2) with the (3) and rearranging the equation, we obtain the momentum equation considering the particle velocity at the state 0 u0=0.

P1-P0=ρ0u1U .(10)

Generally speaking, for a generic case of u we obtain:

P1-P0=ρ0(u1-u0)(U-u0) .(11)

### Energy Balance

The implication of the energy balance is that the rate of energy increase of the control mass is equal to the rate of work being on it. The rate of work done on the control mass would be the change in pressure-volume product divided by the time required for the process. The volume divided by time is the same as area times velocity; so the rate at which work is done on the control mass is:

wt=P1Au1-P0Au0

The rate of energy increase of the control mass is the sum of the rate of change of internal energy plus rate of change of kinetic energy.

The internal energy, E, is the mass times the specific internal energy, e.

Therefore, the rate of change of internal energy is:

Et=(ρ1AL1e1-ρ0AL0e0)/t

The rate of change of kinetic energy is:

KEt=(12ρ1AL1u12-12ρ0AL0u02)/t

Developing the calculations following the statements written her above and combining the equation with the mass one (2) and momentum one (4), we get the energy equation, and as before, we are going to consider it with u0=0.

e1-e0=12(P1+P0)(v0-v1). (12)

Where for a generic u0 is:

e1-e0=12P1u1-P0u0ρ0U-u0-12(u12-u02) (13)

Now, we have three equations for the five variables. In order to solve shock problems, we need two more relationships.

## The Hugoniot Planes

As written in paragraph 3.3.3, we still need two more equations, now we will see that these are derived from experimental data of material properties and from specification of boundary conditions. These new relationships are called the Hugoniot equations.

### The Hugoniot equation

We found in the previous session that five basic parameters were involved to descrive fully, and therefore calculate, a shock wave: P (pressure), U (shock velocity, u (particle velocity), ρ (density), and e (specific internal energy). We had derived the three equations based upon mass (8), momentum (11), and energy conservation (13).

Where, the subscripts 0 and 1 refer to the states just in front of and just behind the shock front, respectively. What we need at this point is another relationship relating these same parameters. One such relationship is called an equation of state (EOS). The EOS gives all of the equilibrium states in which a material can exists and is written in terms of specific internal energy, pressure, and specific volume. We do not have a general EOS that can be derived for all materials, unfortunately. There is, of course, the ideal gas equation, PV=Nrt, where RT is related to the specific internal energy, however we are not dealing with ideal gases though. We are interested in solids. If there were such an EOS, e=f(P,v) then it could be combined with the energy jump equation and the energy, e, term could be eliminated, giving us the relationship P=f(v). This is the Hugoniot equation. In the mass and momentum equations we have two equations in four variables (the final states plus U). If we could determine some relationship involving any two of these that was also specifically descriptive of the states of a material, then we would have an alternative to the EOS-derived Hugoniot. Out of the four variables we could determine a Hugoniot equation relating any two. The choices will be among the following:

P-U, P-u, P-v, U-u, U-v, and u-v.

In the past, many experiments were conducted to determine such a relationship, and it was found that the shock velocity was linearly related to the particle velocity, for most materials.

U=C0+su (14)

So, a Hugoniot equation could be determined from experimental data. Out of the possivle six variable pair “planes” mentioned above, we are going to take advantage of three of them: U-u, P-v, and P-u.

### The U-u plane

The first value of the U-u relationship is linear. Therefore, we determine U and u experimentally for a particular material, at a number of shock states, we could plot the data on this plane, strike a straight line through the data points and easily find the U-u Hugoniot equation. This is shown in Figure 15 below. The equation for this linear relationship is again U=C0+su. The constant C0 is called the bulk sound speed, but this is misleading. C0 has no real physical meaning other than the fact that it is the y-axis intercept on a straight line drawn through the data points. U, C0 are expressed in km/s and the term s is dimensionless. Unfortunately, there is no simple correlation of the constants C0 or s with any other material properties. Therefore, it is possible not to find the Hugoniot for a material in any references, the only thing we can do is use the U-u Hugoniot values of another material that is similar to the one of interest. Try to match the materials by chemical and physical type, that is, same chemical family and similar crystal structure and habit. Also try to match by mechanical state, that is, a pressed powder, a casting, a foam, etc.

Figure 15- U-u Hugoniot data.

### The P-v Plane

If we combine the U-u Hugoniot equation with momentum and mass equations, and let P0=0 and u0=0, we can eliminate the particle and shock velocity terms and get an expression P=f(v).

P=C02v0-v[v0-sv0-v]-2 (15)

This is the Hugoniot in the P-v plane. Plotted, it is as appears in Figure 16 down here. The isentrope, the path function that describes a continuity and not a jump, is different from the Hugoniot. Remember that a relief wave is a continuous process and its path would be along the “unloading” isentrope. Since the Hugoniot represents the locus of all possivle states behind the shock front, then a line joining the initial and final states on the P-v Hugoniot represents the jump condition. This line is called the Raleigh line and is shown in Figure 16 as well. If we eliminate the particle velocity term u by manipulating the mass and momentum-jump equations, and let u0=0, we get:

P1-P0=U2v0-U2v1v02 (16)

This is the equation of the Raleigh line, and we see that the slope of this line is dP/dv0, namely, U2/v02. If we know the initial and final P-v states of shock, then we can calculate the shock velocity by taking the slope of the Raleigh line. We have now fixed the fifth variable by specifying it as a boundary condition in our calculation. Another consideration that we would like to point out, the straight segment at the lower ed of the Hugoniot diagram in Figure 16 (under point A) is the familiar linear elastic stress-strain relationship and P0 is the elastic limit. Furthermore, if we shock a material, which means going through the Raleigh line from the state P0,v0 to Pb,vb and allow a relief wave to bring it back to P0,v0  (passing through the Hugoniot curve) there will be a change in specific internal energy. Thus, in the process of shocking and then relieving the material, we increased the final specific internal energy by the amount equal to the difference between the area under the Raleigh line and the area under the Hugoniot segment. Changes in internal energy are changes in thermal state; therefore, the material must have increased in temperature; it got hot.

Figure 16- Typical P-v Hugoniot

### The P-u Plane

Again, starting with the momentum and U-u Hugoniot equations, this time we will manipulate the equations to eliminate U, leaving P-u

P0=0, u0=0;

P1=ρ0u1U

U=C0+su

P1= ρ0u1(C0+su1)

Now let us bring another factor into play. Until now we have looked at calculations on the P-v Hugoniot, always allowing u0, the initial material particle velocity, to be zero. This, in effect, meant looking at the shock, relative to the material, in Lagrangian terms. Now, allow the material to be in motion before shock arrival, that is, u0≠0, by changing u1 (Lagrangian) to (u0- u1), the Eulerian transform for the particle velocity.

P1= ρ0u1(C0+su1) (Lagrangian) (17)

P1= ρ0C0(u1-u0)+ ρ0s(u1+u0)2(Eulerian)

Figure 17-P-u Hugoniots

The next implication we must note is that, since we are dealing with velocity on one axis, we must remember that velocity is a vector quantity; it has direction indeed. What we described and by inference have done in Figure 17 and in Eq. 17 was plot the Hugoniot for a material where the shock travels from left to right (a right-going shock). When the shock travels from right to left, then in the convention we established above, the particle velocity would be -(u1-u0). For that case, the Hugoniot for a right-going shock in Eq. 17 would not be correct. In order to find the Hugoniot for a left-going shock, let us go back again to the P-u plane and change (u1-u0) in -(u1-u0).

PR= ρ0C0(u1-u0)+ρ0s(u1+u0)2

Right-going.

PL= ρ0C0(u0-u1)+ρ0s(u0-u1)2

Left-going. It is the left-to-right mirror image of the right-going Hugoniot, and it too “slides” back and forth along the u axis depending upon the value of u0. The slope of the line connecting two states, before and behind a shock, on the left-going Hugoniot is -ρ0(U-u0), where the minus sign tells us that the shock is moving towards the left.

This property will allow us to solve interaction of shocks, which we will see in the next paragraph and everything will be clarified.

## Shock wave attenuation, rarefaction waves

As anticipated in paragraph 3.2, the rarefaction phenomenon attenuates the pressure behind the shock front. Thus, let us say something more about this important phenomenon without looking at its development, which is plenty complicated since we have to deal with thermochemistry and thermodynamics equations as well as recall the Hugoniot eqs.

First of all we are going to find “rarefaction” under different names such as relief wave,  unloading wave, and Taylor wave. The rarefaction wave is the progression of particles being accelerated away from a compressed or shocked zone. It travels in the direction opposite to the acceleration of the particles. This is opposite to a shock wave, where the particles are accelerated in the direction of the shock. Unlike a shock wave, which is nearly a discontinuity and is very steady, a rarefaction wave is spread out in space, and continues to spread with time. This is because the expansion of the high-density material to a lower density takes time. In other words, the velocity of a rarefaction wave is dependent on time. In brief, the basic characteristics of a rarefaction wave can be concluded in the following:

- A rarefaction wave can be formed in various situations. For example, (1) in an explosive detonation, immediately following a shock wave, a rarefaction wave is usually formed since the shocked material is to release; (2) when a shock wave travels from a high shock impedance material to a low impedance one, a rarefaction wave occurs since the pressure in the high impedance material is reduced; (3) when a shock wave propagates to a free surface, a rarefaction wave is caused.

- The unloading of a rarefaction wave is along a continuum (isentrope) that is approximated by the Hugoniot. Because of this property, a problem relevant to a rarefaction wave interaction can be solved by using the method in shock wave interaction, described previously.

- The velocity of a rarefaction wave varies with time since the unloading is along the Hugoniot curve.

- The particle-moving direction in a rarefaction wave is always opposite to the direction of the rarefaction wave. In contrast, the particle movements and the shock wave are always in the same direction.

- A rarefaction wave, either in its form or in its pressure, can be affected by different factors such as material properties, boundary conditions, etc. In rock blasting, these factors include explosive properties, stemming, charge parameters (such as charge length, primer placement, etc.), and rock properties.

### Experimental result for shock wave attenuation in different materials

Similar to elastic and plastic stress waves, shock waves in various media due to different attenuation mechanisms such as irreversible compression, plastic deformation, phase change, rarefaction, and geometrical attenuation. As a result of the interaction between rarefaction waves and other attenuation mechanisms the shock waves generated in solids have a decreasing velocity till the shock waves become elastic waves.

Consider an explosively generated shock wave in an inert specimen. Since the mass of the explosive is finite, the pressure behind the shock front falls off rapidly due to rarefaction waves. The peak pressure decays as the wave propagate outward because the energy distributed over the constantly increasing area and also is due to finite dissipation of energy in the transition through shock front. Figure 18(left) shows a test set up for loading the shock waves in solids by contact explosive detonation and Figure 18(right) shows progressive attenuation of the peak shock pressure with distance travelled in the solid specimen. The peak pressure and the shock velocity decreases till the shock wave becomes an elastic wave. Since the sound velocity is increasing function of pressure, the shock pulse becomes broader with distance travelled in solid specimen.

Figure 18-Test set up (left), Attenuation of peak pressure P with the distance travelled in solid specimen (right)

The decrease of pressure of shock wave in travelling a unit distance in the solids is dependent on the pressure gradient in non-uniform wave profile behind the shock front and the ratio of velocities of rarefaction wave and the shock wave in the specimen. If Us is the velocity of the shock front then pressure can be calculated from shock jump condition

P1=ρ0u1U

Shock velocity and particle velocity are related by a linear equation

U=C0+su

The pressure attenuation with distance is written as:

dPdx=(up+cU-1)dPds (18)

where c is sound velocity in compressed solid, dP is change in pressure over a distance dx in solids and dP/ds is pressure gradient behind the shock front. The pressure gradient dP/ds is a function of the length of explosive. In the near region of shock loading the shock velocity attenuates according to an exponential law:

U=U0e-αx(19)

Where U0 is the amplitude of transmitted shock velocity at the explosive solid interface and is attenuation constant, α is the attenuaton coefficient and depends on the material, incident shock velocity and shape of the shock front.

We can set the thing in terms of pressure as well, namely Eq. 19 describes simply an exponential decaying, which is already well known from our experiment, since the shock velocity U is strictly linked to the pressure P, it is possible to express the same law:

P=P0e-αx (20)

## Shock propagation from one material to another

When P0 and u0 are equal to zero, from the momentum equation we have P=ρ0uU. The product ρ0U is called the shock impedance, similar to the characteristic impedance of an elastic medium. The shock impedance can be expressed by the letter Z, that is, Z=Uρ. When a shock wave propagates from material A (left, blue) to material B (right, green), three cases are dealt with: (1) ZA<ZB (2) ZA>ZB and (3) ZA=ZB The examples for these cases are: (1) in deck-charge blasting when shock waves travel from one deck to another; (2) in normal blasting when shock waves propagate from the explosive to stemming; (3) in normal blasting when shock waves go into rock mass from the blasthole. We are going to analyse in particular the (2) case, which is suitable for our purpose, as well as useful to make us understand the propagation of the shock wave through stemming, and thus, to get the shock wave pressure which can reach another detonator placed in another generic deck, as shown in Figure 1 .

When a shock wave in material A travels into B through their interface, there are also two more shock waves to be caused: one reflected wave going back to A from the interface, and one shock wave going into material B, as seen in 3.4.4. Correspondingly, there are three Hugoniot curves. The wave going back into material A is reducing its pressure from P1 down to P2. This is a rarefaction wave rather than a shock wave. In this case, the reflected wave is diffuse. In other words, in the P–u plane the curve from the original shock state in material A goes to lower pressure. It is formed at pressure P2 and is going into B, raising the material in B to a new state 2. The Hugoniot curve for this shock in B is over the Hugoniot curve for the original shock in A, as shown in Figure 19, since the shock impedance of B is greater than that of A. It is an isentrope, not a Hugoniot curve. It can be calculated if the material properties are known, or measured by experiments.

Figure 19- Example of shock wave travelling in two material; ZA>ZB

The isentrope centered at the shock state and the Hugoniot curve centered at the same state are tangent at second order, and make a smooth curve through the centering point. In many cases, particularly with metals that are not too different in shock impedance, for the reflected rarefaction wave, we can use the Hugoniot curve for the reflected shock to replace the isentrope for the rarefaction wave as an approximation. Figure 19 shows the three Hugoniot curves and the solution of the problem. As a short conclusion, we find that when a shock wave travels from a high to low impedance, the shock wave does decrease in pressure. In contrast, the particle velocity increases.

How to solve such a problem?

First step:

P1= ρ0A u1(C0A+sA u1) Blue Vurve

Having the pressure P1 we can get easily the particle velocity u1 from the equation up here.

Second step:

The Hugoniot for the new shock (or reflected shock) traveling in the negative x direction in the “donor” is:

PL= ρ0C0(u0-u)+ρ0s(u0-u)2 Red Curve.  Indeed left-going.

Where u0=2u1

Thus, we can obtain the Hugoniot curve for reflected shock in A. The Hugoniot for the new shock traveling in the positive x direction, in the stemming is:

P2= ρ0B u(C0B+sB u) Green Curve.

Therefore, we can get P2 and u2 which will be less than the original values at state 1. Easy peasy.

## Underwater explosions

What we have just read in paragraph 3.5 and 3.6 is well known and it can lead us to the belief of having all the answers, thus towards the fact that we can rule the shock pressure through these equations. Unfortunately, it is still hard to provide a certain relationship that links all these phenomena in a real application case, especially in ours.

However, it would be interesting to have a look at a different environment such as underwater, in which we can, in somehow, simplify the our problems, though, always taking into account that this kind of experiments are valid only in water.

There are many aspects of the underwater explosion event that must be studied in order to properly understand the development and propagation of the dynamic shock loading through the fluid.

The following sections of this paper provide an introduction to the basic features associated with the underwater detonation of an explosive charge. These features include: the formation of the shock wave and its effects and shock wave refraction effects.?????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????????

# Pressure Sensors

## Introduction

Experimental measurement of shock wave pressure by means of pressure gages is not an easy task to accomplish. Explosive discontinuities, temperature effects, shock wave effects (electromagnetic frequency pulses), embedment material of the gage, etc., have been responsible of contributing to ambiguous measurement results. There has been, however, a variety of piezo‐resistive and piezo‐electric sensors that have been used to measure detonation pressure including manganin gages, carbon gages, tourmaline gages, quartz crystals, lithium niobate crystals, piezo polymeric film gages (PPF) and carbon composition resistors (CCR). Selection of the appropriate gage will depend not only on the pressure range to be measured, but also on cost, the latter an important consideration given the destructive nature of the experiments. From the above choices, two fairly different gages were selected: the piezo‐resistive carbon composition resistors and the PPF piezo‐electric film gages. The former, are provided by the company “Allen Bradley”, which nowadays are out of production, were used as the principal gage for this investigation, given their low cost, availability and particularly their acceptance as a reliable sensor for harsh non‐homogeneous environments. On the other hand, PPF film gages, in particular the FDT series provided by “TE connectivity”, were used in the second place, since their behaviour has never been studied and analysed for shock pressure measurements within a borehole, indeed has not still presented a calibration curve or useful piece of information regarding their application in our field.

## Carbon composition resistors

The carbon resistor gauge has previously been studied by numerous researchers in several different initial resistances values. This gauge is essentially a simple carbon composition resistor that can be used as a pressure gauge with little or no modification. The only equipment needed is a power supply to provide a small amount of constant current (signal conditioner).

Figure 20- Pressure measurement circuit by means of a signal conditioner (current generator).

Because of the ease of use, ability to measure pressures in the range up to 3-5 GPa, and survivability in harsh environments, it can be used in cases where no other gauge would survive. However, because the gauge is manufactured to act simply as a resistor and not as a pressure gauge, a calibration that is empirical in nature would seem evident. Because of this, the gauge is generally described as having only approximately 15% accuracy. Past experiments at Lawrence Livermore National Laboratory (LLNL) have incorporated using the 470 Ω carbon resistor gauge in energetic materials that often make measurements in the range of 3-5 GPa and at times in the low-pressure regime less than 0.4 GPa. The resistors used in this work were standard 1/8 Watt, 470 Ω carbon composition resistors made by Allen-Bradley Corporation. The nominal dimensions of the resistor are 1.7 mm diameter and 4 mm long, with wire leads extending from each side of the length of the cylinder, as showed in Figure 20, 21 (you can find all the details, in its datasheet, attached in Apendix????????????????????????????).

Figure 21- Carbon Resistor by Allen Bradley, Data-Sheet

Figure 22- Carbon Resistor by Allen Bradley.

Unluckily this kind of resistors/sensors are nowadays out of production, though, since Queen’s University own one of the last batch, we had the possibility to obtain and analyse data. This was the main reason, which led us to focus our attention to the other kind of sensors, that so far, nobody has collected useful pressure data inside boreholes.

However, it is still possible to find these resistors on the market, though the price has increased so much and now you might found them for about 20$ each.

### Working principle

The basic principle of the piezoresistive pressure sensor is to use a strain gauge made from a conductive material that changes its electrical resistance when it is stretched. The change in resistance is converted to an output signal.

There are three separate effects that contribute to the change in resistance of a conductor:

- The resistance of a conductor is proportional to its length so stretching increases the resistance.

- As the conductor is stretched, its cross-sectional area is reduced, which also increases the resistance.

- The inherent resistivity of some materials increases when it is stretched.

The latter, namely the piezoresistive effect, varies greatly between materials. The sensitivity is specified by the gauge factor, which is defined as the relative resistance change divided by the strain:

GF=∆RRε

Where ΔR is the change in strain gauge resistance due to axial strain and lateral strain, R is unstrained resistance of the strain gauge and finally, ε is the strain, namely ε=ΔL/L0 (change in length over original length)

In the end, reminding the first Ohm’s law I=VR;

V(R)=R(t)I (21); as we can see in Figure 23.

### Calibration Equation Selection

Various experimental arrangements have been used by researchers to calibrate the pressure response of carbon composition resistors; these responses being measured in terms of relative resistance and relative conductance (inverse resistance) change. Ginsberg et al (1991) used gas gun and aquarium experiments to produce calibration equations, Hollenberg (1986) used shock wave experiments in water for his calibration experiments while Wieland (1987 and 1993) proposed a calibration constant relating pressure and relative conductance change that was claimed to work well for pressure amplitudes below 1.0 Kbar. What they obtained are the following formulas, namely a relationship between ΔR and pressure P.

Wieland:

Pkbar=R0-RR10.2 (22)

For 0<P<1.0 kbar

Ginsberg et al:

PGPa=7.001-4.345RR0+0.364R0R-8.40e-(RR0) (23)

For 0<P<50kbar

In the above equations, R0 and R refer to the original and the instantaneous resistances respectively, expressed in ohms.

However, for this work we are going to take advantage of the Katsabani’s formula, which is a good compromise, speaking in terms of pressure, since it has a restricted range of pressure, namely for 0<P<3kbar.

PMPa=4848.08(R0-RR02+94.15R0-RR00.5) (24)

In the end, what we collected from the sensor, is an answer in terms of voltage as we can see in the following figures:

Figure 23- Voltage Drop Due To  Shock Wave

Figure 24-Pressure Curve Obtained From Figure 19

## Piezo polymeric film gages

Transducer materials convert one form of energy into another, and are widely used in sensing applications. The tremendous growth in the use of microprocessors has propelled the demand for sensors in diverse applications. Today, piezoelectric polymer sensors are among the fastest growing of the technologies within the $18 billion worldwide sensor market. Like any new technology, there have been an extraordinary number of applications where "piezo film" has been considered for the sensor solution. In this paragraph we will focus only on one of thousands of applications, which in our case would as pressure sensor.

Piezoelectric materials undergo a dipole deformation and electric charges accumulation in response to applied mechanical stress. An electric field will generate and thus voltage can be detected on the upper and lower surface of the piezoelectric materials. When stress is removed, the voltage will disappear. This phenomenon is called direct piezoelectric effect.

The sensor, showed in Figure 25, is flexible, lightweight, tough engineering plastic available in a variety of different sizes, thicknesses and shapes. Its properties as a transducer include:

- Wide frequency range—0.001 Hz to 109 Hz.

- Vast dynamic range (10-8 to 106 psi or µ torr to Mbar).

- Low acoustic impedance—close match to water, human tissue and adhesive systems.

- High elastic compliance.

- High voltage output (10mV-100V).

- High dielectric strength—withstanding strong fields (75V/µm).

- High mechanical strength and impact resistance (109-1010 Pascal).

- High stability—resisting moisture.

Figure 25-FDT series

However, the major disadvantage of piezoelectric pressure sensors is that they can only measure dynamic or changing events. It is because a static pressure will result in a fixed amount of charges on the piezoelectric material, causing an initial voltage output.

As written in the introduction of Chapter 4 we used a particular kind of sensors produced by “TE connectivity” and the price for each sensor is nowadays about 4$, for these reasons they became very suitable for our research.

Since we did not find any information about their composition, either in its DataSheet, we had to ask the company directly. That is what Richard Brown, Application Manager at TE connectivity, Sensors’, says about the sensors we are going to use for the last part of this thesis.

“Our FDT series of piezo film elements are constructed using PVDF (polyvinylidene fluoride) polymer which we process in-house to make it piezoelectric.  This polymer film is then printed on each surface with conductive (silver) ink patterns to provide an electrode structure.  Finally, the electrodes are coated with a very thin clear acrylic coating to prevent the formation of Ag2S (tarnish).”

(you can find other details, in its datasheet, attached in Appendix????????????????????????????).

### Working principle

Piezoelectricity arises from the cross-coupling of mechanical and electrical energy within certain materials; these materials can be used as both actuators and sensors. When a piezoelectric device is used as sensors, it actively generates charges in response to external loads, as compared to piezoresistive sensors where a voltage/current source is applied across the sensor to measure the strain.

Figure 26- Piezoelectric effect sketch considering PZT as piezoelectric material

Therefore, looking at  Figure 25 we can see a piezoelectric crystal (PZT), which is placed between two metal plates, however, in our sensors, the piezoelectrive material is PVDF, polyvinylidene fluoride.

At this point the material is in perfect balance and does not conduct an electric current.

Mechanical pressure is then applied to the material by the metal plates, which forces the electric charges within the crystal out of balance. Excess negative and positive charges appear on opposite sides of the crystal face.

The metal plate collects these charges, which can be used to produce a voltage signal, which is going to be treated and analysed throughout different pieces of equipment.

# Field Experiments

## Introduction

"The experiment serves two purposes, often independent one from the other: it allows the observation of new facts, hitherto either unsuspected, or not yet well defined; and it determines whether a working hypothesis fits the world of observable facts."-Rene J. Dubos.

In the following chapter, we are going to explain our analysis methodology, as well as pieces of equipment and besides, there will be included different methodologies of acquiring data, which we considered most appropriated for our purposes. We carried out experiments according to different methodologies as well as equipment, however, we kept improving our measurements, in almost each experiment experience, through the change of the sensor design and recording data; this is the reason why the Reader will find different paragraphs regarding the same kind of sensors but made with a different construction method and different instrumentation.

## Site characterization

## CCR Gauges

As reported in paragraph 4.2, carbon composition resistor are just resistors that change their resistance according to the stress they are exposed to. The design of these sensors was about the same for all the versions; the resistor was connected, in “somehow”, to a coaxial cable, which is shown in Figure 27. However, as written in 5.1, we used different methodologies, therefore in Figure 28 it is possible to take a look at the other cable used in the third typology of sensors. Thus, as you can see we have two main cables, the centre conductor and the braid, they will be connected to a Signal Conditioner on one side, on the other side instead, they will be linked to our  resistor as positive and negative pin.

Figure 27- Generic coaxial cable cutaway view

Figure 28- Twisted cables with one wire for each

The resistor is well known and its function is well explained 4.2.1, we designed three different sensors, where, the first two are pretty similar, instead the last one is completely different and it responded in a better way to our experiments.

### CCR, first typology

The first experiments were done throughout this typology of CCR. The resistor is connected to the centre conductor by means of a “copper tube” which has been squeezed and well coupled using simply a plier, then, in order to insulate the first pit, a rubber tube was shrunk (Figure 29)

Figure 29- First step, CCR, first typology

After this, it was connected the other pin to the braid cable, which it was twisted as you can see in Figure 29. Now, in this first case, we soldered those two with some tin and then we placed another piece of rubber tube and we shrank it as well. At the end of these two steps, we obtained our “raw sensors”, Figure 30.

Figure 30- Second step, CCR, first typology

Before sealing the whole sensor into a pipette full of mineral oil, we must check the resistance, since we heated up (to shrink the rubber tubes). We noticed that in this way we influenced the resistor itself and we got a change of a bunch of Ohms, about 1-15Ω, both heating it up and soldering it. These were the main reasons why we decided to change a bit the design in the other experiments.

As anticipated, the sensor will be placed inside a dropper full of mineral oil. This gives us different advantages, such as the possibility to have a sterile environment for the resistor as well as physical protection, and least but not last, a shield against Electro Magnetic Fields. The latter was a big problem in past experiments, since that the analysis of the data has been made impossible due to EMF, which produced too much noise and thus, it was not possible to extrapolate suitable data.

Figure 31- Finished sensors, first typology

In the Figure 31 up here, it is possible to see the finished sensors before their use on the site. As you can see, the pipettes are tied up with some plastic ties and besides, the sensor has been placed inside them trying to avoid, as much as possible, formation of air bubbles, which could affect considerably our pressure measurement. Furthermore, the avoiding of air bubble was one of the main challenge in this design; first because some air remained inside the rubber shrink, since it was not too adherent to the resistance whereas that we wanted to keep the temperature (by means of a heat-gun) as low as possible in order not to affect the resistance itself. Second, because we could not shrink the pipette since the mineral oil could have caught fire.

### CCR, second typology

Basically the construction process was almost the same, however as it is said in paragraph 5.3.1, the change of resistance value could have been avoided without using both the solder and the last shrink (the main one that cover completely the sensor, which besides will decrease the probability of  making air bubbles). Therefore, the first step is shown in Figure 29, as well as for the first typology of sensor, in the end the sensors will result in the way of Figure 32 down here though.

Figure 32- Finished sensors, second typology, not sealed yet (always through using plastic ties)

As you can see from Figure 32, the resistance is in direct contact with the mineral oil. It is possible to observe the main differences between these two kinds of sensors just looking at Figure 31-32.

The first and second type have been used for different experiments but unluckily they were not so suitable for boreholes (the signal was plenty noisy), for this reason we used them to analyse the shock wave behaviour in water, using: buckets, garbage bins and pipes.

### CCR, third type

The third typology instead, were used for nearly all of the experiments, this was due to the fact that they responded in a good way to the pressure pulse. They were used in both buckets and boreholes, as well as in both the environments, we obtained clear pressure curves.

Speaking of the construction method, it was quite different since in this kind of sensors there will not be any pipette and mineral oil as well.

Figure 33- CCR, third typology, before being insulated

The main difference., as is possible to notice in the first place, is the cable to which the sensors is connected (the yellow one), indeed is much thinner compared to the coaxial one visible in Figure 29-32. This feature will allow us to get a clearer signal with less noise and furthermore, less interferences and loses of the shock wave pulse which will travel through it. Then the resistor is connected to the cables through two pieces of copper tube well pushed together, and finally, one of the pin was shrank by means of a heat-gun, taking care of the resistor that has not to be warmed too much.

In the end, we dipped the resistor (showed in Figure 33) into a naphtha rubber solvent for making it insulated. This rubber solvent has a good tensile strength, as well as great dielectric strength proprieties, and besides, it avoided the problem regarding air bubbles attached to the sensor. (Data sheet in Appendix ????????????????????????????????????????????????????????????????)

Figure 34- Finished sensor, third typology, insulated with a special coating.

## CCR Experiments

The experiments that we are going to talk about, took place at the site described in paragraph 5.2. We ran different kind of experiments in order to understand, as much as possible, the limits, as well as the main features of this technology. What we observed was remarkable, even though it took a bit more than we expected since we have been some problems with the instrumentations, which will be explained further.

First of all we will introduce the first tests, dated 20/09/2019, then we will arrive till the last one, which was ran in date 11/10/2019.

### First test setup 13/09/2019

As you can see in Figure 35, the pieces of equipment used were mainly two: the signal conditioner, which in first place allows us to supply the sensors with a current of 4mA (in this case) and in second place it provides us the output signal, the other tool that you can observe in the picture is the “MicroTrap” and we are going to describe in brief this electronic box.

Figure 35- First test’s setup, starting from the left side, rightwards: Signal conditioner, Microtrap, Multimeter, Chargers.

This piece of equipment is supposed to work in this way, as indeed written in part of its Data Sheet down here:

Number of Channels: Standard = 1 VOD channel. Optional = 1 VOD + 4 Scope channels.

Resolution: 14 bits, 1 part in 16,384.

Recording Rates: Selectable from 1 Hz to 2 MHz.

Non-Volatile Memory: Standard = 4 million data points. Optional = 8 million data points.

Recording Time (@ 2 MHz Recording Rate): Standard = 2 seconds. Optional = 4 seconds.

Trigger Mode: Trigger internally on the signal from the event (2 to 98%) or trigger externally from a trigger wire.

Multiple Event Storage: 1 to 16 tests can be stored in the internal memory.

Power: AC mains or internal rechargeable NiCad battery, which provides 12 hours of active operation on a full charge. Charger provided.

Size and Weight: 21 x 17 x 9 cm (8.3 x 6.7 x 3.5 in.) 2 kg (4.4 lbs.).

Environmental: Fully operational at -40 to +60 °C (-40 to +140 °F). Snow, rain, dust and sand proof. Drop proof from a height of at least 1 m (3 ft.).

Basically this instrument allows us to bring just one little box to the site instead of different oscilloscopes, since usually they have 2 channels each (nevertheless, it is possible to purchase oscilloscopes with 4 channels and more, but they are going to be much more expensive) and furthermore, the Microtrap has been designed for acquiring data either in extreme situations, as it is possible to read in its “Environmental properties”.

Figure 36- Pressure curve. TEST 1, 30mm faraway from the primer

Figure 37- Voltage drop, TEST 1, 30mm faraway from the primer

In Figure 36-37 is shown Channel 1 of the first test we performed. We wanted to see the sensor’s behaviour (described in 5.3.1, first typology) in an ideal environment, in our case a cylindrical bucket filled with normal water.

#### Experiments Description

First of all we will describe the different part of the experiment, as it is possible to observe in Figure 38.

As explosive we used a primer of Pentolite 50/50 TNT/PETN (Table 1, by Orica) for more information, included in the DataSheet, check the Appendix??????????????????????????????????????

| Nominal Diameter | 60 mm |
| --- | --- |
| Nominal Length | 120 mm |
| Nominal Mass | 454 g (1 lb) |
| Shell Material | Open Top Cardboard Canister |
| Shell Colour | Fluorescent Orange |
| Nominal Density | 1.6 g/cm3 |
| Nominal Detonation Pressure | 25 GPa |

Table 1- Pentolite Technical Data

The ignition system was a sort of mix between the electrical system and the NONEL one. Essentially a Blasting Machine is linked to an electrical detonator which ignites the NONEL system, which means that the electrical detonator is well connected to a shock tube, as it is possible to see in Figure 37 (the yellow one), that is connected, in its turn, to a NONEL detonator placed inside the primer. We took care placing the electric detonator, since it could have produced EMF and so, noise during the experiment, for this reason we located the electric detonator as far as possible from the sensors and we buried it in order to confine as much as possible its explosion.

Furthermore, in Figure 38 is possible to notice the presence of two cables twisted to each other and situated inside the explosive. Those cables will work as “Trigger line” for the Microtrap/DataTrap (which is essentially a faster Microtrap). Therefore, when the detonation begins its own reaction, those cables will melt together and thus, the DataTrap will start recording data.

Finally, our sensors (first type) are hanged by means of plastic ties and besides, they are well bonded to a piece of wood.

The bucket has dimensions: 44.3 cm height, 30cm top diameter, 26cm bottom diameter, and it was filled until reached about 34cm.

Figure 38- Experiment 1, Primer on the bottom of the bucket, sensors hanged at difference height from the top primer.

Nevertheless, we found out that this machine does not work in the way in which they affirm. This means that: after an accurate checking on this machine (we would like to point out that this checking was done on the Signal Conditioner [S.C.] as well, since in the first place we blamed the S.C.), Prof. Katsabanis and Prof. Rielo noticed that the signal output response was not coherent. Indeed, through a wave generator, they produced a square pulse signal, as input, and as output, the Datatrap provided them a “rounded square pulse”, this could be due to the fact that the machine interpolates data to build a smooth output signal. Furthermore, Prof. Katsabanis checked some past field data obtained by the Datatrap and in fact, they showed an mismatch of about 20-15% with respect of the laboratory data. This gap might be due to the Datatrap since it is proved that it could provide a range of uncertain pretty similar because of its probable interpolation.

This above, was the reason why all the following experiments were carried out using 2-4 oscilloscopes as signal recorders, which they will give us the exact signal as output. The all results of the experiments concerning the same kind of sensors, the same design are visible in Appedix???????????????????????, considering that the distances from the top of the primer change in order to have different points (we ran three different experiments that day, using the same methodology as well as instrumentation.

### Second test setup 27/10/2019

In this second experience, we ran different tests concerning the same buckets we read in 5.4.1.1, as well as a bin (simply a bigger bucket) and a pipe, thus the following paragraphs will regard these three different kind of tests carried out using always the same primer explosive, by Orica, with same technical data visible in Table 1. However, this time we used different pieces of recording equipment, as indeed anticipated in the last paragraph; we used two Tektronix TDS1001B oscilloscopes (two channels each) and one Yokogawa DL750 scope (four channels).

| Frequency | 40, 60, 100 MHz |
| --- | --- |
| Sample Rates | Up to 1 GS/s real time |
| Channels | 2 |

Table 2- Tektronix TDS1001B main features

| Sample Rates | Recording Time | Channels |
| --- | --- | --- |
| 10 MS/s | 100 s | 4 |
| 1 1 MS/s | 600 s |  |
| 100 KS/s | 9000 s |  |
| 10 KS/s | 72000 s |  |
| 1 KS/s | 864000 s |  |
| 200 S/s | 2592000 s |  |

Table 3- Yokogawa DL750 oscilloscope, main features

Figure 39- Tektronix TDS10001B (left), Yokogawa DL750 (right)

Thus, in total we have eight channels, which means eight sensors. Therefore, we had to add another S.C. In addition, this time we set them for 10mA of current output, in this way we obtained a clearer signal and obviously a high voltage drop, though, remember that current is taken into account in Eq. 21, 24, the pressure curve will be drawn in the same way, since V(R), R(t).

#### Buckets

FARE LO SKETCH, GUARDARE QUADERNO, METTERE TABELLE DA EXCELL

## PPF Gauges

## PPF Experiments

https://qspace.library.queensu.ca/bitstream/handle/1974/907/Silva_Guillermo_C_200711_PhD.pdf?sequence=1&isAllowed=y

https://www.sciencedirect.com/topics/engineering/underwater-explosions

https://asa.scitation.org/doi/full/10.1121/1.4892668?crawler=true&